Give your webcam

Infrared Vision!

Web cams and digital cameras tend to have filters behind their lenses to remove “noise” caused by infrared radiation. Here we look at a way to remove this filter from a particular model of webcam. Our guide is based on materials from www.mutr.co.uk who supplied the webcams.

Note that this will not turn your webcam into a thermal imaging camera. The wavelengths emitted by objects at body temperature are too long to be detected. Nevertheless, you will be able to carry out many interesting experiments and activities.

Step 1: Loosen the screw at the back of the webcam.

Step 2: Prise the front off.

Your webcam should now be in two parts.

Step 3: Unscrew the lens

Step 4: Remove the filter from the lens assembly. Don’t worry if it breaks.

Reassembly is a straightforward reversal of disassembly.