# Fitting the Accelerometer to a PASCO cart

The adaptor plate can be fixed to other types of trolley using self-tapping screws.

Fit the adaptor plate to the cart exactly as shown in the picture on the right. Note the long bolt that sticks up through the plate.

The cart has a plunger at one end. The long bolt should be nearest this end.

Fit the accelerometer to the adaptor as shown below.

All being well, the X arrow on the accelerometer should be pointing to the back of the trolley (towards the plunger).

We have used this technique to attach it to a remote-controlled car.